Zen (microarchitecture) from AMD

Zen is the codename for a computer processor microarchitecture from AMD, and was first used with their Ryzen series of CPUs in February 2017. The first Zen-based preview system was demonstrated at E3 2016, and first substantially detailed at an event hosted a block away from the Intel Developer Forum 2016. The first Zen-based CPUs codenamed "Summit Ridge" reached the market in early March 2017, Zen-derived Epyc server processors launched in June 2017 and Zen-based APUs arrived in November 2017.

Zen is a clean sheet design that differs from the long-standing Bulldozer architecture. Zen-based processors use a 14 nm FinFETprocess, are reportedly more energy efficient, and can execute significantly more instructions per cycle. SMT has been introduced, allowing each core to run two threads. The cache system has also been redesigned, making the L1 cache write-back. Zen processors use three different sockets: desktop and mobile Ryzen chips use the AM4 socket, bringing DDR4 support; the high-end desktop Zen-based Threadripper chips support quad-channel DDR4 RAM and offer 64 PCIe 3.0 lanes (vs 24 lanes), using the TR4 socket and Epyc server processors offer 128 PCI 3.0 lanes and octal-channel DDR4 using the SP3 socket.

| L1 cache | 64 KiB instruction, 32 KiB data per core |
| --- | --- |
| L2 cache | 512 KiB per core |
| L3 cache | 8 MiB per quad-core CCX |

History and development

AMD began planning the Zen microarchitecture shortly after re-hiring Jim Keller in August 2012.AMD formally revealed Zen in 2015.

The team in charge of Zen was led by Keller (who left in September 2015 after a 3-year tenure) and AMD Senior Fellow and Chief Architect Michael Clark.

Advantages over predecessors

### Manufacturing process

Processors based on Zen use 14 nm FinFET silicon. These processors are being produced at GlobalFoundries, though reports state some Zen processors may also be produced at TSMC. Prior to Zen, AMD's smallest process size was 28 nm, as utilized by their Steamroller and Excavator microarchitectures.

### Performance

The Zen processors also employ sensors across the chip to dynamically scale frequency and voltage. This allows for the maximum frequency to be dynamically and automatically defined by the processor itself based upon available cooling.

### Memory

Zen supports DDR4 memory (up to eight channels) and ECC.

Pre-release reports stated APUs using the Zen architecture would also support High Bandwidth Memory (HBM). However, the first demonstrated APU did not use HBM. Previous APUs from AMD relied on shared memory for both the GPU and the CPU.

### Power consumption and heat output

Processors built at the 14 nm node on FinFET silicon should show reduced power consumption and therefore heat over their 28 nm and 32 nm non-FinFET predecessors (for equivalent designs), or be more computationally powerful at equivalent heat output/power consumption.

### Enhanced security and virtualization support

Zen added support for AMD's Secure Memory Encryption (SME) and AMD's Secure Encrypted Virtualization (SEV). Secure Memory Encryption is real-time memory encryption done per page table entry. Encryption occurs on a hardware AES engine and keys are managed by the onboard "Security" Processor (ARM Cortex-A5) at boot time to encrypt each page, allowing any DDR4 memory (including non-volatile varieties) to be encrypted.

The next architecture after zen is zen +

| AMD Zen+ |  |
| --- | --- |
| Produced | April 2018 |
| Designed by | AMD |
| Common manufacturer(s) | GlobalFoundries |
| Cores | 4-6 (mainstream) 8 (performance) 12-32 (enthusiast)[1] |
| L1 cache | 64 KiB instruction, 32 KiB data per core |
| L2 cache | 512 KiB per core |
| L3 cache | 8 MiB per quad-core CCX |
| Transistors | 12 nm (FinFET) |
| Socket(s) | Socket AM4 Socket TR4 |
| Predecessor | Zen |
| Successor | Zen 2 |
| Product code name(s) | Pinnacle Ridge (Desktop)[2] Colfax (HEDT)[1] Picasso (APU) |
| Brand name(s) | Ryzen Ryzen Threadripper |

The next architecture after zen+ is zen2

| AMD Zen 2 |  |
| --- | --- |
| Produced | 2019 Q1/Q2 (estimated) |
| Designed by | AMD |
| Common manufacturer(s) | TSMC (core die) Global Foundries (I/O die) |
| Min. feature size | 7 nm[1][2] |
| Cores | up to 64 (server) |
| Socket(s) | Socket AM4 Socket TR4 Socket SP3 |
| Predecessor | Zen+ |
| Successor | Zen 3[3][2] |
| Product code name(s) | Matisse (Desktop) Castle Peak (HEDT) Rome (Server)[2] |

Ryzen

It is a brand of x64 and x86 microprocessors designed and marketed by AMD (Advanced Micro Devices) for desktop, mobile and embedded platform based on Zen and Zen+ microarchitectures. It consists of central processing units marketed for mainstream, enthusiast and workstation segments and accelerated processing units (APUs) which is marketed for mainstream and entry-level segments and embedded applications.

Тasks of laboratory work

1. Read from the Zen-microarchitecture

2. Fill in the table for ZEN:

Аrchitecture of the AMD company

| Architecture name (Code Name) | Date (year) | Fab | Socket | Cache | Support instructions |
| --- | --- | --- | --- | --- | --- |
| Zen |  |  |  |  |  |
| Zen+ |  |  |  |  |  |
| Zen2 |  |  |  |  |  |

3. Аnswer the teacher's questions:

ZEN is a microarchitecture or processor brand?

How many cores are in Zen-architecture?

Which architecture is more productive: Zen, Zen+ or Zen2? Why?